TECHNICAL INFORMATION FOR ECO IMPREGNIR L

TYPE:			High viscosity methyl silicone solution

DELIVERY

SHAPE: 		The solution in organic solvents

FEATURES

AND USE:	ECO IMPREGNIR L is used for the protection of wood and wooden surfaces. It prevents the occurrence of mould and other microorganism on wood i.e. prevents wood rotting. It is also used as an anti-woodworm tool and against the creation of larvae in the wood.

ECO IMPREGNIR L is long-lasting material based on special silicones, and unlike other materials that have the same application it is completely harmless and non-toxic.

Along with ECO IMPREGNIR M, means for the protection of metals and metal alloys from the corrosion, it is commonly used for conservation of antique (trophy) weapons and tools.

PREPARATION FOR

APPLICATION: 	ECO IMPREGNIR L is applied in a delivered shape. The wood intended for protection should be dry and clean.

APPLICATION

METHOD: 	ECO IMPREGNIR L is applied to the wood by spraying or coating using pneumatic sprayer, mop, sponge etc. The procedure is performed several times to material saturation.

If the wood has been attacked by larvae (worm-eaten) after spraying to saturation it is necessary to wrap it in foil (PVC, PE, PP) and keep it wrapped for 5-15 days. After removal of the foil apply ECO IMPREGNIR L to the wood by spraying or coating using pneumatic sprayer, mop sponge etc. in one layer to material saturation. The wood is protected after the complete evaporation of solvent.

ECO IMPREGNIR L consumption: 0,10-0,50L/m2.

SAFEGUARD

MEASURES: 	ECO IMPREGNIR L is the solution in organic solvents, it is flammable, so its application must comply with fire safety measures, as when applying similar products that contain flammable solvents. For professional use only!

PACKAGING: 	200L Metal barrels, 10L plastic cans and 1L plastic bottles.

STORAGE:	ECO IMPREGNIR L should be stored outdoors and indoors away from the heat sources.

PRODUCT

LIFETIME:	ECO IMPREGNIR L can be kept in the original packaging for 6 (six) months.