TECHNICAL INFORMATION FOR ECO RECEPT 01

TYPE:			Acidic solution of surfactants

DELIVERY

SHAPE: 		Single component water system

FEATURES

AND USE:	ECO RECEPT 01 is used to remove limescale and other mineral efflorescence (aka saltpeter) from the stone, artificial stone, mortar, concrete, brick and similar materials. Along with alkaline detergent ECO RECEPT 02 it is used to remove graffiti from the walls of buildings.

Along with detergent ECO RECEPT 04 is used to remove corrosion from the surface of metals and alloys.

PREPARATION FOR

APPLICATION: 	Before the use ECO RECEPT 01 should be diluted with cold water in a volume ratio of 1:10 to 1:3 (medium : water), depending on the type and degree of dirt of the area that needs to be cleaned.

APPLICATION

METHOD: 	ECO RECEPT 01 is applied to the surface by spraying with pneumatic sprayer or brush coating. After a while the surface is rinsed with water. In the case of very dirty surfaces procedure should be repeated.

Special attention should be paid when cleaning the limestone and similar materials to which high concentration detergent can cause damage.

SAFEGUARD

MEASURES:	ECO RECEPT 01 is highly acidic detergent, so it is necessary to use the appropriate safety equipment. In case the concentrated detergent comes in contact with exposed skin wash the spot with large amount of cold water. If accidentally gets into the eyes flush them immediately with cold water. Also, the mandatory intervention of a doctor is required (washing with sodium bicarbonate solution). Surfaces that are accidentally sprayed with this means should be washed with water immediately in order to avoid damage. For professional use only!

PACKAGING: 	10L Plastic cans and 1L plastic bottles.

STORAGE:	ECO RECEPT 01 should be stored outdoors and indoors at temperatures above 4oC.

PRODUCT

LIFETIME:	ECO RECEPT 01 can be kept in the original packaging for 6 (six) months.