TECHNICAL INFORMATION FOR ECO RECEPT 02

TYPE:			Alkaline solution of surfactants

DELIVERY

SHAPE: 		Single component water system

FEATURES

AND USE:	ECO RECEPT 02 is used to remove graffiti from facades and for cleaning different excessively greasy surfaces. ECO RECEPT 02 dissolves and removes the carbonized remains of mineral oils, edible oils and other grease, as well as carbonized remains of starch and sugar.

ECO RECEPT 02 is used to remove moss, fungus, algae, lichens and other organic matter from building materials during repairs, restoration and conservation work.

PREPARATION FOR

APPLICATION: 	ECO RECEPT 02 is applied in a delivered shape. Depending on the type and degree of dirt on the area that needs to be cleaned ECO RECEPT 02 should be diluted with cold water in a volume ratio of 1:10 to 1:3 (medium : water).

APPLICATION

METHOD: 	ECO RECEPT 02 is applied to the surface by spraying with pneumatic sprayer or coating with brush or cotton cloth.

Concentrated ECO RECEPT 02 is used to remove graffiti from facades. After a while, when the paint starts to dissolve, the surface is rinsed with water, sometimes along with mechanical cleaning with steel brush and clean cotton cloth. The procedure is repeated until the desired result is achieved.

ECO RECEPT 02 diluted to 1:10, 1:5 or 1:3 is used for cleaning dirt from facades, floors, windows etc. After a while the surface is rinsed with water.

ECO RECEPT 02 diluted 1:5 to 1:10 used for cleaning carbonized remains of organic matter from facades and other surfaces. After a while surface is rinsed with water under pressure.

In the case of very dirty surfaces the procedure is repeated several times.

We recommend the use of acidic detergent ECO RECEPT 01 which is applied to the surface after rinsing ECO RECEPT 02.

SAFEGUARD

MEASURES:	ECO RECEPT 02 is very alkaline detergent, so it is necessary to use the appropriate safety equipment. In case the concentrated detergent comes in contact with exposed skin wash the spot with large amount of cold water. If it accidentally gets into the eyes flush them immediately with cold water. Also, mandatory intervention of a doctor is required (washing with boric acid solution). Surfaces that are accidentally sprayed with this means should be washed with water immediately in order to avoid damage. For professional use only!

PACKAGING: 	10L Plastic cans and 1L plastic bottles.

STORAGE:	ECO RECEPT 02 is stored outdoors and indoors at temperatures above 4oC.

PRODUCT

LIFETIME:	ECO RECEPT 02 can be kept in the original packaging for 12 (twelve) months.