TECHNICAL INFORMATION FOR ECO RECEPT 04

TYPE:			Acidic solution of surfactants

DELIVERY

SHAPE: 		Single component water system

FEATURES

AND USE:	ECO RECEPT 04 is used to remove corrosion from the surface of metals and metal alloys. It is primarily used for cleaning rust from iron and steel.

ECO RECEPT 04 also presents one type of metal pre-protection. After drying of the means any standard color for metal can be applied on the treated surface.

When is necessary to keep the original color of metal RECEPT ECO 04 is used along with ECO IMPREGNIR M, means for the protection of metals and metal alloys from the corrosion.

PREPARATION FOR

APPLICATION: 	Before the use ECO RECEPT 04 should be diluted with cold water in a volume ratio of 1:5, 1:4 or 1:3 (medium : water), depending on the amount of corrosion that needs to be removed.

APPLICATION

METHOD: 	ECO RECEPT 04 is applied to the metal by spraying with pneumatic sprayer or coating with brush or sponge. After applying the means surfaces that are much corroded should be rinsed with water and wiped, then treated again.

SAFEGUARD

MEASURES: 	ECO RECEPT 04 is acidic detergent, so it is necessary to use the appropriate safety equipment. In case the concentrated detergent comes in contact with exposed skin wash the spot with cold water. If it accidentally gets into the eyes flush them immediately with cold water. Rinsing with sodium bicarbonate solution is also recommended. Surfaces that are accidentally sprayed with this means should be washed with water. For professional use only!

PACKAGING: 	10L Plastic cans and 1L plastic bottles.

STORAGE:	ECO RECEPT 04 is stored outdoors and indoors at temperatures above 4oC.

PRODUCT

LIFETIME:	ECO RECEPT 04 can be kept in the original packaging for 12 (twelve) months.